# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Equipment for Renovation at Hotel Simsek in Merkez, Edirne, Türkiye	p 1 / 5

Publication reference: BGTR0300155-PP2-SUP-01/LOT 2

LOT 2: Simple renovation work in hotel rooms

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 2 | Simple renovation work in hotel rooms. |  |  |  |
| 2.1. | Installation of energy control systems for 40 team rooms: The system, designed to ensure efficient climate control in a 40-room facility, should be based on the principle of using battery-powered, low-energy wireless sensors mounted on balconies or window sashes, communicating with a room-based or central control unit. Each component in the system should be uniquely identified to prevent signal interference. An automatic "Shutdown" command should be sent to the air conditioner after a 30-120 second delay when a door or window is opened to stop unnecessary energy consumption, and the air conditioner should reactivate in its previous operating mode when the door is closed. Climate control modules should have the ability to learn infrared signals regardless of the brand of existing devices, the status of the entire system (on/off/battery level) should be monitored instantly by the reception or technical unit through a central interface, and the installation process should be completed wirelessly in a way that does not damage the existing structure, aesthetics or cabling infrastructure. |  |  |  |
| 2.2. | Dismantling, delivery, and installation of 36 sets of windowed doors for 4 floors. This work involves the installation, renovation, dismantling (demounting), transportation to the specified address, and reassembly (installation) of a total of 36 units (Doors, Sliding Doors, Balcony Door and Window Sets, Interior Room Doors, Hotel Main Entrance Door, and Garden Gate) located within the hotel. Work Distribution for the 36 Door Units: 18 Units: Installation of Sliding PVC Balcony Door Systems, 10 Units: Renovation of Room Balcony PVC Door and Window Sets, 6 Units: Renovation of Interior Room Doors, 1 Unit: Renovation of Hotel Main Entrance Door, 1 Unit: Renovation of Garden Gate. Disassembly (Removal) Principles: The disassembly process will not cause any damage to the building facade, interior walls, or floor coverings. Existing sashes and glass panels must be removed from their frames to prevent breakage and deformation. Each disassembled unit should be labeled with its floor and room number to avoid confusion during reassembly. The contractor will sweep up any large debris resulting from the demolition, leaving the area clean. Shipping and Delivery The vertical transportation of units between floors and their horizontal transportation within the vehicles is the responsibility of the contractor. Necessary protective measures (such as using wedges, blankets, stretch wrap, etc.) will be taken to prevent the glass from cracking and the PVC profiles from scratching during transport. |  |  |  |
|  | Assembly and Application Criteria PVC Sliding and Balcony Systems Balance Control: All systems will be installed on the scale and plumb line. There should be no contraction, strain or spontaneous movement in the sashes. Insulation: The gap between the window frame and the wall will be filled with polyurethane foam; the interior and exterior surfaces will be sealed with high-quality neutral silicone to prevent air and water tightness. Equipment: Espag mechanisms, levers and sliding system wheels will be delivered lubricated and adjusted. Interior Room Doors Door frames should be fully fixed to the wall, and moldings should be mounted so that there are no gaps at the joints. The locking mechanisms and latch should function smoothly and without friction. Main Entrance and Garden Gates Main Entrance Door: Hinge settings should be adjusted according to hotel entry traffic, and if applicable, the door closer (hydraulic) should be calibrated according to the speed of passage. Garden Gate: Since it will be exposed to outdoor weather conditions, the mounting elements must be made of stainless steel, and the ground connections must be made with maximum strength. |  |  |  |
| 2.3 | Group 1: Restoration of partially damaged exterior plaster during dismantling and installation of a door with a window. -After door installation, any loose parts, dust, and mortar residue resulting from removal will be completely cleaned from the surface. -The surface to be plastered will be slightly moistened to increase the adhesion of the new plaster. -The joints between the door frame and the wall will be reinforced with plaster mesh to prevent future cracks caused by expansion and contraction. -The applied plaster will be gauged to match the existing facade texture. Along the line where the plaster and the window frame meet, a suitable filler will be applied to prevent water leaks and provide complete protection. |  |  |  |